# This document

This is a monthly report submitted by the National Center for Atmospheric Research (NCAR) describing progress on tasks listed in the annual statement of work (SOW) for the 4DWX project, funded by the U. S. Army Test and Evaluation Command’s (ATEC).  Descriptions of tasks are abbreviated.  For full descriptions, please refer to the SOW.  The sections, subsections, and tasks in this document are numbered to match those in the SOW.  Ten reports are submitted per year; none is submitted in the two months (usually April and August) during which project-review meetings (i.e., IPRs) are held.

This report is for work done in June 2011.

# Themes and goals for the fiscal year

During FY11, NCAR is striving to preserve the current system's operational reliability and to make advances in science and engineering.  NCAR continues to customize 4DWX for each range by optimizing the configuration of the NWP core of the system (e.g., number of vertical levels) and by exploring ways of improving or adding to the coupled applications driven by the NWP model’s output. Additional instances of AutoNowcaster (ANC) are being installed, following on the great success of the first ATEC-funded installation of an ANC at WSMR.

This is the third year of a multi-year reduction in the resources spent on graphical applications; NCAR is increasingly relying on third-party applications, for which NCAR provides data translators. The 4DWX Web Portal will continue to be the ranges’ chief interface to pre generated, operational weather maps and plots from 4DWX.  IDV is replacing JViz as the chief tool for more detailed exploration of weather data from 4DWX.

Sophisticated and more user-friendly methods of model verification are one of the project’s

foci in FY11. Verification efforts will be applied toward tools that can aid forecasters in real

time.

FY11 is the second in a multi-year effort to consolidate 4DWX hardware at two sites, one

primary, the other as a back-up to the primary site.

# Budget summary

Please see the SOW for FY2011 (separate document).

# Restrictions and security

Please see the SOW for FY2011 (separate document).

# Programmatic tasks

## Project reviews

- IPRs
Report:	  The next IPR will be 23–24 August in the Damon Room at the Mesa Lab, with the RDT&E meeting on the 25th at the Foothills Lab in room FL0-2512.  The picnic will be at Foothills Community Park on Wednesday evening, the 24th.  Rooms and picnic shelters have been reserved.  The pre-IPR meeting will be 3 August in FL0-2512.

## Forecaster Training

  - Forecaster Training sessions at NCAR
Report:	  Sessions were held on 29–31 March and 10–12 May 2011.  Each session included a visit to the NWS office in Boulder.  A summary of the sessions and results of exit polls will be presented at the IPR in August.

## Internships

  - ATEC internships
Report:  No work.

## Security

- Adoption of CACs

Report:  Recent tests of a USB reader on Windows computers were successful.

- Compliance with International Traffic in Arms Regulations (ITAR)

Report:  Completed.

- Training and certification of systems administrators

Report:  Completed.

# Technical tasks

## Support

  - Maintenance of Help Desk

Report:  Standard support is ongoing.

  - Maintenance of 4DWX Knowledge Base

Report:  The new liaison continued reviewing the 4DWX documentation.  Material is being evaluated to decide what needs to be updated and improved.

  - Teleconferences with ranges

Report:  All-range teleconferences are normally scheduled each month to discuss problems and requests for individual ranges as well as topics of interest to everyone.  Over the past several months, these meetings were convened, with participation from each range listed:

- April 6:	ATC, CRTC, DPG, RTC

- May:		No telecon due to April IPR

- June 8:		ATC, CRTC, DPG, RTC, WSMR, YPG

  - Support for safari missions (see Appendix B for a full list of missions)

Report:  Support was provided for the Safari mission listed in Appendix B.  The lone mission during June was a continuation of the San Nicholas Island configuration for WSMR.  Normal support included the monitoring of runs and trouble-shooting of run failures.

  - Demonstrations of real-time support for tethered systems in theater
Report:  No longer a task assigned to this FY.

  - Proposals for acquisition of dedicated or shared HPCs

Report:  NCAR worked with Dell engineering staff to architect a new HPC system for purchase using $675K provided by the Meteorology Division of the WDTC.  NCAR has finalized the quotes and will purchase the system in July.

  - Preparation of material for Range Commanders Conferences

Report:  No work.

  - Hire of range liaison

Report:  Completed.

  - Upgrade of the operating system on the HPC at DPG

Report:  Completed.

## Customization of 4DWX for each range

    - Evaluation and enhancement of 4DWX for simulation of extreme cold at CRTC

Report:  Test simulations are being run to explore the chinook (aka foehn) winds that are sometimes responsible for the most dramatic breaks in very cold weather at CRTC.  The simulations so far are simply designed to test a few run-time configurations, including one with higher vertical resolution.  Preliminary results, if ready, might be shown at the IPR in August.

    - Improvement of forecasts of temperature at ATC, including cold air damming

Report:  Work is underway to more closely match 4DWX’s land/water mask to the actual observation sites and coastlines on and near the range.  Roughly a dozen input fields depend on this specification, and all have to be modified to maintain self-consistency.  Current tests are focused on a higher-resolution geographic dataset that has become available, which more closely matches the coast of the Chesapeake Bay.

    - Improvement of gravity wave forecasts at WSMR

Report:  No work.  Progress on this awaits some initial results from item D below (misc. optimizations).

    - Miscellaneous additional testing and optimization of 4DWX configurations

Report:  Initial trial simulations continue on chinooks at CRTC, microphysics and lightning prediction at RTC, and land-use resolution and characterization at ATC.

## User applications and data management

- Gridded bias correction of 4DWX output

Report:  NCAR continued pursuing whether there is still a book-keeping error in management of data files from model output.  The effort was paused while engineers addressed more pressing tasks.

- Incremental upgrades to 4DWX Web Portal

Report:  Development continued for a 4DWX patch release targeted for late summer.  Feedback from the Forecaster Training sessions is being used to guide changes made.

- Maintenance and enhancement of MIR database

Report:  No work.

- Replacement of JViz with IDV

Report:  Development continued for a 4DWX patch release targeted for the summer.  Feedback from the Forecaster Training sessions is being used to guide changes made.

- Upgrade of data gateway systems

Report:  Work on the data gateway systems was focused on three areas in June: 1) specifying a hardware replacement for the primary gateway, whose 5-year warranty recently expired, 2) continuing development of the automated failover mechanism, 3) development of technical support documentation.

- Evaluation of SIPS as a possible replacement for NAPS

Report:  No work.  Dependent on schedule of SIPS development by NSWC.

- Finalization and freeze of Lewis Model at NCAR

Report:  NCAR staff traveled onsite to WSMR in June and discussed the Lewis model workflows and requirements with the forecasters.  A follow up teleconference was then held to establish POCs in both groups and specific action items.  Based on that teleconference, WSMR will provide examples to NCAR, and NCAR will make the software changes in Lewis to fully support WSMR’s workflows, including file formatting and removal of rotated-wind calculations.  Once these changes are implemented, verified, and deployed, the Lewis software at NCAR will be considered complete.

- Other general upgrades to applications

Report:  NCAR is in the final stages of updating the RIP namelist-creation code in response to input provided during user testing at the most recent Forecaster Training.  The code has been fully documented so it can be moved from the responsibility of developers to the range liaison, who has begun his training on the RIP software.

## NWP modeling

- Upgrades to WRF Model in 4DWX

Report:  Development continued on minor updates to the 4DWX modeling software.  These updates will be combined with release of the community WRF 3.3 model after 4DWX enhancements are made.  The new version of 4DWX modeling software are planned for deployment in autumn 2011.

- Refinement of nudging scheme in 4DWX

Report:  No work.

- Development of hybrid data-assimilation scheme for 4DWX  (multi-year effort)

Report:  We continue to test the RTFDDA-3DVAR hybrid system with radar data assimilation for ATC domains in a real-time environment. Monitoring and assessment are ongoing.  It has been found that latent heat nudging (LHN) with hourly radar-data-updates improves short-term forecasts of storm structures and rainfall, while nudging 3DVAR analyses produces mixed (positive and negative) effects.  Off-line case studies are also being conducted in order to produce improved 3DVAR analyses.  It was found that when the latent heat adjustment is incorporated in the background fields, better 3DVAR analyses are obtained.  Assimilating these better 3DVAR analyses in RTFDDA clearly improves 0–6 h rainfall forecasts.  The results were presented at the 12th WRF Users' Workshop held in June 2011 in Boulder, CO.  Additional testing will be conducted in the next few months.

Development of the 4D-REKF (4-Dimensional Relaxation Kalman Filter data assimilation and forecasting) also continues with improved memory handling by reconstructing the 4D-REKF Kalman gain processing algorithms. All of the code has been written and we are testing it.  Recent modifications removed a memory bottle-neck.  We are continuing the real-time tests of 4D-REKF with the new RTFDDA v3.2.1-0 to evaluate 4D-REKF and compare with RTFDDA; results were reported in several papers at the 12th WRF Users Workshop.

- Demonstration of E-4DWX at ranges other than DPG

Report:  No work.

- Refinement of E-4DWX perturbation schemes

Report:  New PBL schemes have been added to E-4DWX and we are preparing a major upgrade. The new energy backscattering perturbation scheme newly available in the community WRF v3.3 will be examined and probably implemented later this calendar year.

- Extension of E-4DWX calibration to more variables and to full model grid (multi-year effort)

Report:  NCAR is tracking down a bug to the verification algorithms employed in the automatic post-processing procedure that generates the calibrated values for near-surface variables.

- Operational implementation of MODIS-based SSTs and LSTs in 4DWX

Report:  Python scripts have now been rewritten to accommodate changes in the latest Python libraries.  Work is underway to modify configurations of the WRF Model to test SST updates during simulations, not simply at cold starts and hot starts.

- Other general improvements to 4DWX

Report: We have continued to implement and test the new on-line observation quality control scheme. This work is still in progress. We are implementing algorithms in the WRF code to interpolate the missing data needed for data assimilation. This part was done in the old off-line data QC module. At present we are testing the new QC algorithm as an off/on-line interim solution before we solve some technical issues in order to input the diverse raw data.

- Exploration of LES modeling at one or more ATEC ranges

Report: The tornado outbreak in the southeastern U.S. in April 2011 has been simulated with the WRF-RTFDDA-LES system.  We are analyzing the output.  Preliminary results were presented at the 12th WRF Users’ Workshop, and it impressed the audience.  Surface winds reach Enhanced Fujita Scale 4 (>166mph) about 10 miles southeast of Decatur.  Most essential ingredients in tornado genesis (strong inward spiral flow, updrafts and downdrafts) are pretty clearly depicted by the modeling system. The simulation demonstrates the capability and robustness of the WRF-RTFDDA-LES system in capturing fine-scale extreme weather. The figure below is a zoomed-in sub-area (40 x 40 km) of the huge model’s LES domain. RTC is just a bit out of the picture (NE corner).  A paper based on the simulation is under preparation.

Figure 1.  Simulated tornado-like windstorms on 27th April 2011 5:10 CDT near Decatur, Alabama.  Vectors are surface wind (reference vector is Enhanced Fujita Scale 0). Colored dots mark surface wind speeds that reached the Enhanced Fujita Scale 0 (29.1 m/s) or above.  Colored contours are 700-m vertical velocities.

To prepare for future implementation of WRF-RTFDDA-LES system at the ATEC ranges, 30-m ultra-high resolution land-use data for the conterminous United States have been collected and processed for WRF-RTFDDA-LES.  The raw data were released in February 2011 (National Land Cover Data, NLCD 2006) by USGS, while the current WRF system uses NCLD 1992 at 1-km resolution. This is the first national land cover product at 30 meter spatial resolution. We will test and validate these ultra-high resolution land cover data for ATEC ranges in the next half year.

## Auto-Nowcaster and convective weather forecasting

- Installation of full ANC at RTC

Report:  The ANC installation at RTC was accomplished in March.  RTC's NOAAport remains offline, so satellite and surface data (from projects at NCAR) are being pushed to the RTC ANC. Scott Halvorson is working to solve this problem.

NCAR staff traveled to RTC from 6-8 July. Changes were made to the ANC display, several software problems were identified and solved, and one problem with the lifted index field is still being resolved. We provided additional training to RTC on fuzzy-logic interest fields related to convection initiation forecasts.

At NCAR, archival of the RTC data files to the mass storage system is being implemented.

- Implementation of probabilistic forecasts of lightning at WSMR and RTC

Report:  NCAR is validating the radar-based short term time lightning potential forecast and is examining performance statistics.  We have debugged the RTC model-based lightning potential and are computing statistics for the summer of 2011 to examine performance skill.

NCAR staff continue to work with Chris Schultz, UAH, who is visiting until September and is working on the LMA lightning jump algorithm.

- Maintenance and operational support of ANC and related tools at WSMR and other ranges

Report:  The installation of new hardware for the ANC was completed in late June, and the implementation and set-up of the ANC system was completed by in late July. NCAR staff are traveling to WSMR the week of July 25th to check out the new system and to turn off the old one. Both old and new systems are running for now.

There is a problem with the satellite ingest at WSMR that Scott Halvorson is looking into. Much of the data are dropped.

The install of new MRRD DAS hardware was finalized.

A bug was corrected in the ingest of level three radar data for the MRRD that only showed up if an azimuth was represented as being 360 (as opposed to 0).

## Model validation and verification

- Development of more sophisticated, user-friendly methods of model verification
Report:  No work.

- Operational verification of E-4DWX
Report:  No work.

- Development of a simple graphical tool for viewing 4DWX’s verification statistics
Report:  Because this work was paused for some time, a meeting was held to review the previous use cases and design criteria.  Engineering has resumed.

- VV&A of the WRF-based 4DWX to satisfy army requirements

Report:  No work.

## Tasks funded through other sources but leveraged to benefit ATEC-4DWX project

- Use of 4DWX or E-4DWX to support forecasting at Cape Canaveral.  (Currently unfunded and indefinitely on hold.)

Report:  No work.

## Hardware, operating systems, and security

- Expansion of data storage

Report:  Latest expansion is completed.

- New desktop and laptop computers for project staff

Report:  None.

- Consolidation of operational 4DWX hardware

Report:  Completed.

- Purchase and maintenance of R&D clusters at NCAR

Report:  Warranty extensions and operating system license renewals were purchased for the smac-c3 cluster, which is used for safari mission model runs and 4DWX modeling research.

- Purchase and maintenance of DAS hardware and miscellaneous upgrades to DAS software

Report:  New 2-node DAS systems were purchased and shipped to DPG, ATC, RTC, and WSMR.  These systems, and a new WSMR ANC system, were successfully installed and set up during onsite range visits by NCAR staff in June.  All 7 ranges now have similar configurations with two independent DAS machines.  The DAS1 systems receive output products from Big MAC A and the DAS2 systems are fed from Big MAC B.  This architecture provides a continuously available backup mechanism that 4DWX users may select themselves when either of the Big MACs is unavailable.

## Management

- NCAR’s project management

Report:  No change.

## Administration and miscellany

  - Articles, posters, and talks

Report:

- Articles published

  - None

- Articles accepted and in press

  - Delle Monache, L., T. Nipen, Y. Liu, G. Roux, R. Stull, 2011: Kalman filter and analog schemes to post-process numerical weather predictions. Conditionally accepted on Monthly Weather Review.

- Articles in revision based on reviews

  - Hirschberg , P., Abrams, E., Bleistein, A., Bua, W.,  Delle Monache, L., Dulong, T., Gaynor, J., Glahn, B., Hamill, T., Hansen, J., Hilderbrand, D., Hoffman, R., Morrow, B., Philips, B., Sokich, J., Stuart, N., 2011: A weather and climate enterprise strategic implementation plan for generating and communicating forecast uncertainty information. Bulletin of the American Meteorological Society, conditionally accepted.

- Articles in external review

  - None.

- Articles submitted

  - None.

- Articles in internal review

  - None.

- Articles in preparation

  - Hirschberg , P., E. Abrams, A. Bleistein, W. Bua, L. Delle Monache, T. Dulong, J. Gaynor, B. Glahn, T. Hamill, J. Hansen, D. Hilderbrand, R. Hoffman, B. Morrow, B. Philips, J. Sokich, and N. Stuart, 2011: A weather and climate enterprise strategic implementation plan for generating and communicating forecast uncertainty information. To be submitted to the Bulletin of the American Meteorological Society.

  - Hopson, T. M. and J. P. Hacker, 2011: Combined approaches for ensemble post-processing, Mon. Weath. Rev., in process idea: application of quantile-regression approach to NOAA Reforecast temperature ensembles over Utah.

    - Status: some calculations are in the process of being redone using an improved logistic regression selection technique; most figures completed, 70% of text.

  - Hopson, T. M., Assessing the ensemble forecast spread and skill relationship, Mon. Weath. Rev., in process.

    - Status: rewriting text and including material from 2 new references

  - Article on cold air damming at ATC and sensitivity of forecasts to PBL and microphysics schemes

  - Liu, Y., J. C. Knievel, T. M. Hopson, S. F. Halvorson, J. C. Pace, G. Roux, W. Wu, J. P. Hacker, S. E. Krippner, T. T. Warner, and S. P. Swerdlin, 2012:  Mesoscale ensemble weather prediction at Dugway Proving Ground.  Mon. Wea. Rev., in preparation

  - Liu, Y. , L. Pan, Y. Wu, A. Bourgeois, T. Warner, J. Knievel, S. Swerdlin, J. Pace, S. F. Halvorson, and F. Gallagher, 2012: The NCAR 4DREKF ensemble data assimilation and forecasting system: Part 1: System description and validation.  Journal TBD, in preparation

  - Wu, Y., Y. Liu, L. Pan, A. Bourgeois, J. Knievel, T. Warner, S. Swerdlin, S. F. Halvorson, J. Pace, and F. Gallagher, 2012: The NCAR 4DREKF ensemble data assimilation and forecasting system: Part 2: Sensitivity of the critical/control parameters. Journal TBD, in preparation

  - Pan, L., Y. Liu, Y. Wu, A. Bourgeois, J. Knievel, T. Warner, S. Swerdlin, S. F. Halvorson, J. Pace, and F. Gallagher, 2012: The NCAR 4DREKF ensemble data assimilation and forecasting system: Part 3: A Proxy without running ensemble. Journal TBD, in preparation

- Presentations (no order)

  - Presented in June:

      - “4D-REKF: a next-generation FDDA for WRF”. Liu, Yubao, Linlin Pan, Yonghui Wu, Al Bourgeois, Jason Knievel, Tom Warner, Scott Swerdlin, Scott Halvorson, John Pace and Frank Gallagher. 12th WRF Users Workshop. Boulder, CO. June 20 – 24, 2011.

      - “Improve WRF nocturnal LLJ simulation by incorporating turbulence intermittency effect in the PBL parameterizations”. Nagarajan, Badrinath, Yubao Liu, William Y. Y. Cheng, and Thomas T Warner. 12th WRF Users Workshop. Boulder, CO. June 20 – 24, 2011.

      - “Radar data assimilation in the NCAR/RAL RTFDDA system using a hybrid approach of 3DVAR, latent heat adjustment and grid-nudging”. Xu, Mei, Yubao Liu, Wei Yu and Juanzhen Sun. 12th WRF Users Workshop. Boulder, CO. June 20 – 24, 2011.

      - “Simulation of April 22-28 2011 devastating tornado outbreak using WRF-RTFDDA-LES”. Wu, Wanli, Yubao Liu, Ming Ge, Jason Knievel, and Tom. Warner. 12th WRF Users Workshop. Boulder, CO. June 20 – 24, 2011.

      - “Sensitivity Experiments of the NCAR WRF 4D-REKF Data Assimilation and Forecasting System”.   Wu, Yonghui, Yubao Liu, Linlin Pan, and Tom Warner. 12th WRF Users Workshop. Boulder, CO. June 20 – 24, 2011.

      - “WRF 4D-REKF-Proxy FDDA and Initial Tests”. Pan, Linlin, Yubao Liu, Yonghui Wu, Al Bourgeois, and Tom Warner. 12th WRF Users Workshop. Boulder, CO. June 20 – 24, 2011.

APPENDICES

Acronyms

4DWX		Four-Dimensional Data Assimilation and Forecast System

4DWXDB	4DWX database

AGU		American Geophysical Union

AMS		American Meteorological Society

AP		Anomalous propagation

ATEC		U.S. Army Test and Evaluation Command

ATC		Aberdeen Test Center

AUSA		Association of the U. S. Army

CAC		Common Access Card

CAD		Cold-air damming

CFDDA	Climate FDDA

CIDD		Configurable Interactive Data Display

CMAQ	Community Multiscale Air Quality Model

COAMPS	Coupled Ocean/Atmosphere Mesoscale Prediction System

COLIDE 	Convergent Line Detection program

CoN		Certificate of Networthiness

CRTC		Cold Regions Test Center

CVS		Concurrent Versioning System

DART		Data Assimilation Research Testbed

DAS		Data Acquisition Server

DPG		Dugway Proving Ground

EPG		Electronics Proving Ground

ERTFDDA	Ensemble-RTFDDA

GMOD	Global Meteorology on Demand

GPU		Game processing unit

GSL		Great Salt Lake

HPC		High performance computer

HPCMP	HPC Modernization Program

HPSS		High Performance Storage System

IPR		In-process review

ITAR		International Traffic in Arms Regulations

KB		Knowledge Base

LMA		Lightning mapping array

LHN		Latent-heat nudging

LST 		Lake-surface temperature

MIR		Meteorological Information Repository

MIPR		Military Interdepartmental Procurement Request

MODIS	Moderate Resolution Imaging Spectroradiometer

MODTRAN	Moderate resolution atmospheric transmission

MSFC		Marshall Space Flight Center

MSRC		Major Shared Resource Center

MYJ scheme	Mellor-Yamada-Janjic

NAPS		Noise Assessment and Prediction System

NASA		National Aeronautics and Space Administration

NBA		Nonlinear backscatter and anisotropy

NCAR		National Center for Atmospheric Research

NCEP		National Centers for Environmental Prediction

NLDN		National Lightning Detection Network

NRL		Naval Research Laboratory

NSAP		National Security Applications Program (within NCAR RAL)

NSF		National Science Foundation

NVL		Night Vision Laboratory

NWP		Numerical weather prediction

PBL		Planetary boundary layer

R&D		Research and development

RAL		Research Applications Laboratory

RTFDDA	Real-Time Four-Dimensional Data Assimilations system

RTC		Redstone Technical Test Center

SST		Sea-surface temperature

STAR Inst.	Science and Technology in Atmospheric Research Institute

STEP		Short-Term Explicit Prediction

UAH		University of Alabama at Huntsville

UF		Universal format

USWRP	U.S. Weather Research Program

VLAS		Variational Lidar Analysis System

VDRAS	Variational Doppler Radar Analysis System

WRF model	Weather Research and Forecasting Model

WSMR	White Sands Missile Range

YPG		Yuma Proving Ground

YSU		Yonsei University

Safari missions

Report:

Requester:  WSMR

Dates:  Existing SNI configuration running through June

Location:  San Nicholas Island, California

Type:  WRF 3.0.1.1, 3 domains

Special needs:  Support for rocket launches.