Noise - The same as static in a phone line or "snow" in a television

picture, noise is any unwanted electrical signal that interferes with the

image being read and transferred by the imager. There are two main types

of noise associated with CMOS Sensors:

1.

Read Noise (also called temporal noise) - This type of noise occurs

randomly and is generated by the basic noise characteristics of

electronic components. This type of noise looks like the "snow" on a

bad TV reception.

2.

Fixed Pattern Noise (also FPN) - This noise is a result of each

pixel in an imager having its own amplifier. Even though the design

of each amplifier is the same, when manufactured, these amplifiers

may have slightly different offset and gain characteristics. This

means for any picture given, if certain pixels are boosting the

signal for every picture taken, they will create the same pattern

again and again, hence the name.

3.

Blooming - The situation where too many photons are being produced to be

received by a pixel. The pixel overflows and causes the photons to go to

adjacent pixels. Blooming is similar to overexposure in film photography,

except that in digital imaging, the result is a number of vertical and/or

horizontal streaks appearing from the light source in the picture.